## Project Report: Handheld Multi-Sensory Currency Detector

### 1. Introduction

The objective of this project is to design and develop a portable, handheld device capable of identifying currency & its value. This device is specifically engineered to assist the visually impaired and commercial handlers by providing both visual and auditory feedback.

### 2. System Architecture & Components

The device follows a modular design to ensure portability and efficiency. The integration of image processing with audio synthesis allows for a "multi-sensory" user experience.

#### A. Image Acquisition (The Camera)

- Component: CMOS Camera Module (e.g.,ESP32-CAM).

- Role: The camera acts as the primary sensor. It captures a frame of the banknote.

#### B. Central Processing (The Microcontroller)

- Component: High-speed Microcontroller (e.g.,ESP32).

- Role: The "brain" of the device. It performs the following tasks:

  - Image Pre-processing: Converts images to grayscale and resizes them.

  - Pattern Matching: Compares the captured data against stored templates or a lightweight Neural Network.

  - Instruction Routing: Sends the result to both the display and the audio module simultaneously.

#### C. Visual Feedback (The LCD)

- Component: TFT LCD Display.

- Role: Provides a clear visual readout of the currency value (e.g., "$50 USD") and the system status (e.g., "Scanning")

- Design: High-contrast text is used to ensure readability for users with low vision.

#### D. Auditory Feedback (MP3 Module & Speaker)

- Components: DFPlayer Mini (MP3 Module) + 8-ohm Micro Speaker.

- Role: To make the device accessible, an MP3 module stores pre-recorded voice clips for each denomination.

### 3. Power Management & Portability

To ensure the device is truly handheld and practical for daily use, a robust power circuit is integrated:

- Energy Source: A Lithium-Ion (Li-ion) Battery (e.g., 18650 or Li-Po) provides the high current required for image processing.

- Charging System: A TP4056 Charger Module with a USB-C or Micro-USB port allows the user to recharge the device using standard mobile chargers.

- User Control: A physical Toggle Switch or momentary push-button manages the power state, preventing parasitic battery drain when the device is in a pocket or bag.

| Item No. | Item | Price in EGP | Seller |
| --- | --- | --- | --- |
| 1 | ESP32 Camera | 510 | Free Electronics |
| 2 | Perfboard | 25 | Free Electronics |
| 3 | Solder wire | 120 | Free Electronics |
| 4 | IPS Screen 240x240 1.3 inch | 300 | Free Electronics |
| 5 | Switch | 2.5 | Free Electronics |
| 6 | Battery | 120 | Free Electronics |
| 7 | Battery charger TP4056 | 30 | Free Electronics |
| 9 | MP3 TP16 Module | 120 | Free Electronics |
| 10 | Speaker 8 ohm | 12 | Free Electronics |
| 11 | Wires & Headers | 50 | Free Electronics |

Description of each component:

1- ESP32 Camera:

It is an esp32 which is a very famous microcontroller with Wi-Fi and Bluetooth capability also with high clock frequency which is used in processing paired with a camera module used in computer vision applications.

Its main role here is to capture and process images also to broadcast visuals to the LCD and speech to the mp3 module.

2- Perfboard:

It is a breadboard alternative but used for more permanent connection by means of soldering.

3- Solder wire:

It is used to bond wires with the perfboard.

4- IPS Screen 240x240 1.3 inch:

It is used for visuals in this project paired with a microcontroller to handle the graphics and it communicates with the esp32 by means of SPI.

5- Switch:

It is used to complete the circuit upon switching on.

6- Battery:

it is used as a power source.

7- Battery charger TP4056:

Used for charging the battery.

8- MP3 TP16 Module:

It is used for speaker output to read the currency detection output.

9- Speaker 8 ohm:

Used to output sound.

10- Wires & Headers:

Used for connection between components.

Wiring Diagram: